## PROJECT DESIGN DRAWING STANDARDS

The Project Design group uses CADD almost exclusively in the preparation of construction drawings for all projects.  The UPRR Project Design group’s CADD software standard is MicroStation V8i and InRoads.  Consultants preparing drawings for the Project Design group shall, upon the completion of a project, provide a MicroStation-format design file compatible with MicroStation V8i, either on writeable compact disk, via e-mail or the Union Pacific ftp website.

Consultants preparing drawings for the UPRR Project Design Group shall use MicroStation.  Working in another CADD package and translating to MicroStation is not acceptable.

Any consultant preparing drawings for the UPRR will be provided with the UPRR Project Design CAD Configuration Files via the ftp website.  The files are kept current with the latest design standards and are updated as standards change.  It is the responsibility of the consultant to ensure he has the latest version of the files from the site.  The following is a brief description of some of the standard included.

## LEVELS

Each level has a name, number and description.  Color, line style, and line weight, or symbology, is set by the level.

Levels 1 – 63 are Legacy Levels to be used in existing drawings.  Color, line style and line weight are set to 0.  These settings will need to be set to the desired symbology manually, as always.

If the level name begins with…. 	C-	It is a….		Civil Level

G-				General Level

V-				Straight Line Level

If the level name ends with….	-R	Symbology is ….	Remove

-E				Exisiting

-S				Shift

-F				Future

No Letter Designation		Proposed or Construct

## ANNOTATION

## Text & Dimensioning

Notes, Text, and Dimensions

Set appropriate Level, Text Style, Dim Style, Text Height and Line Spacing.

# For levels…	C-ANNO-NOTE			Use…	Text Style = Proposed Text

C-ANNO-NOTE-R			Dimension Style = Construction Dim

C-ANNO-NOTE-S

C-ANNO-NOTE-F

C-ANNO-CONT-NOTE

C-ANNO-CONT-NOTE-R

C-ANNO-CONT-NOTE-S

C-ANNO-CONT-NOTE-F

#### For Level…	C-ANNO-NOTE-E		Use…	Text Style = Existing Text

Dimension Style = Existing Dim

## Text Height and Line Spacing for Various Scales

HT	LS		HT	LS

100 (Default)		10	5		12	6

200			20	10		24	12

50			5	2.5		6	3

40			4	2		4.8	2.4

30			3	1.5		3.6	1.8

20			2	1		2.4	1.2

##### Aerial Notes and Text

### Set appropriate Level, Text Style and Dim Style.

# For levels…	C-ANNO-NOTE			Use…	Text Style = Aerial Text 100

C-ANNO-NOTE-R				   or Aerial Text 200

C-ANNO-NOTE-S			Dimension Style = Construction Dim

C-ANNO-NOTE-F

C-ANNO-CONT-NOTE

C-ANNO-CONT-NOTE-R

C-ANNO-CONT-NOTE-S

C-ANNO-CONT-NOTE-F

###### For Level…	C-ANNO-NOTE-E		Use…	Text Style = Existing Aerial Text 100

or Existing Aerial Text 200

Dimension Style = Existing Dim

Annotation should be as follows:

1234+56.00 24”x36’ CMP MP 34.56
4567+89.00 Pt.Sw. No.20 LH POTO MP 13.45
18+65.00 CL 40’ Conc. Crossing Hwy. 26 DOT# 123 456J MP 123.45

Construction Notes

Construction notes for proposed track , and appurtenances must be placed in boxes for proper shading.

When notes are placed on levels C-ANNO-NOTE, C-ANNO-NOTE-R, C-ANNO-NOTE-S, C-ANNO-NOTE-F, C-ANNO-CONT-NOTE, C-ANNO-CONT-NOTE-R, C-ANNO-CONT-NOTE-S, C-ANNO-CONT-NOTE-F, and either pentable is used, the boxes will block-out anything beneath them when plotted.  When using UPRR TRACK TEXT AND DIM MENU all symbology and text settings are set and the command activated.

## CURVE

Horizontal

For existing curves use cell CRV, or CRVSP, whichever is applicable.

Cell will contain enter-data fields for: Description, (For example, No.1 Main, No.2 Main, Siding), Degree, Total Delta, Spiral Length, and Design Speed

For proposed curves, curve information shall be placed using InRoads Curve Set Annotation command.

Curve Data Placement:

Centered on main curve if possible.

Vetical

Curve Data

Should be placed using InRoads Vertical Alignment Annotation command.

### Profile

Profiles should be placed using InRoads Profile Along Alignment command.

Profile will occupy the top 5” of each full-sized sheet.  Separated from the plan by a horizontal line extending from the left border to the right.  The line should be: LV=G-BORDER

Annotate profiles using InRoads Annotate Profile command.  Place existing elevations along the bottom of the profile, proposed elevations along the top.

## STATIONING

Cardinal (Includes PS, PC, PT, Etc.)

Should be placed using InRoads Horizontal Alignment Stationing command.

Regular (Includes Major, and Minor Stationing)

Should be placed using InRoads Horizontal Alignment Stationing command.

## CELL LIBRARY

Cells shall be placed Relative.

### GENERAl.cel (Contains Consultant Borders and General Sheets)

### HEADWALLS.cel

### MITIGATION.cel

### SIGNAL.cel

### STATE MAPS.cel (To be used on Cover Sheet)

### TRACK CELLS.cel

### TURNOUT TIES.cel

### TURNOUTS.cel (Long tie – Solid line = wood turnout, Dashed line = concrete tie)

### UTILITY.cel

### X-SECTIONS.cel

## MENUBARS

Menubars have been created to simplify the selection of the levels.  Menubars load with a widely available MDL application called barmenu.ma.

### UPRRTrack.mdf – Contains the majority of the design levels.

### UPRRGeneral.mdf – Contains the information needed setting up Sheets.

### UPRRTextDim.mdf – Contains the information on annotation and dimensioning.

### UPRRTurnouts.mdf – Contains UPRR Standrad Turnout cells.

### Straightline.mdf – Levels for creating straightline files.

## PLOTTING

Color Table

### UPRR.tbl

IPLOT Pen Table

HP_1055CM.pen – HP Plotters

CLJ5550.pen – HP Color Laser Jet

SHADING

Use C-ANNO-NOTE, C-ANNO-NOTE-R, C-ANNO-NOTE-S, C-ANNO-NOTE-F, C-ANNO-CONT-NOTE, C-ANNO-CONT-NOTE-R, C-ANNO-CONT-NOTE-S, and C-ANNO-CONT-NOTE-F to shade over information behind notes. Use C-RWAY-REQ to shade Required Right-of-way and C-WETL-IMPACT for Impacted Wetlands these levels will allow elements beneath them to show through.

PLOT SCALE

1” = 100’ preferred.

1” = 200’ if necessary.

## SHEETS

24” x 36” (Full-size) Use cell BORDER, trim on exterior lines.

11” x 17” (Half-size) Use cell BORDER, trim on interior lines.

8 1/2” x 11” (Letter-size) Use cell LBORD.

## ADDITIONAL INFORMATION

### Copy through to drawing, the following survey symbols: (If Desired)

Utilities; Power Poles, Power Lines, Fiber Optic Cables, Water Lines & Meters, Gas Lines.

Signal Cases

Crossings

Culverts

Manholes

Catch Basins

### Track construction lengths will be exclusive of turnout lengths listed below.

No.7 – 84’	No.9 – 107’	No.10 – 120’	No.11 – 125’	No.14 – 176’

No.15 – 180’	No.20 – 230’	No.24 – 264’	No.30 – 374’

### We “Install” turnouts, culverts, road crossings, etc. We “Construct” track.

## Project Design Checklist

- Engineering station, and milepost on all switches. (New construction, and retirements)

- Existing, and proposed signals shown.

- Milepost shown on Intermediate signals.

- Track centers shown on each sheet, and at all transition curves.

- North Arrow placed in any clear area of each sheet.

- Scale bar placed near lower right corner (If Possible) of each sheet.

- Crossing warning devices shown.

- Road names, Crossing surface type, Public or Private, DOT#, Milepost.

- All text LEGIBLE?

- Widen subgrade (6” per inch of Super-Elevation) on outside of curves not having service road on same side.

- Note designating # of “transition ties”.

- “To” Stations on first and last sheets.